DMC 14 Aug 2000

updated 24 Oct 2008

updated 05 Jan 2009, M. Gorelenkova(MG)

*******MG

The distribution function calculation is independent of the NLFBMFLR switch,

and have the code accumulate two copies of the distribution function,

both the particle version (FBM_PTCL) and the guiding center version (FBM) .

We will still use NLFBMFLR, but only to control which copy of the

distribution function is used to compute the beam-beam neutrons

in ../nubeam/bbneut.for and associated routines.

*******MG

The fortran program `get_fbm' has been ported to unix and upgraded

to allow greater flexibility of access to fast ion distributions.

In particular, beam and fusion product distribution functions (now),

and RF tail ion distribution functions (eventually we hope) can be

accessed, at any radial or poloidal location.

Averaging and integration features allow reconstruction of such

quantities as the flux surface averaged total fast ion density

as a function of flux surface label, or, at a particular flux

surface, the poloidal variation of the fast ion density.  Or the

density integration can be restricted to particles within a certain

energy or vpll/v range.  Etc., etc.

The `get_fbm' program can also write the distribution function data

out to a NetCDF file, which will be a more convenient format for

separately written software using this data, than the "encoded ascii"

legacy format used in the .DATAn files output directly by TRANSP.

** a note on units & normalization **

`get_fbm' reads the TRANSP COMMON array fbm from an ACfile (see

Heidbrink's note, below).  The units of FBM are

#/cm3/eV/[delta(solid-angle)/4pi]

but in TRANSP & NUBEAM, "delta(solid-angle)/4pi" is equivalent to

1/[number of vpll/v bins].

------------------------------------------------------------------------

(Discussion of delta(solid-angle) on a unit sphere in velocity space)

In terms of angles on a unit sphere in velocity space:

vpll/v = cos(alpha)

vperp/v = sin(alpha)

delta(vpll/v) = delta(cos(alpha))

For contribution to surface area on the sphare, there is a factor

2*pi*sin(alpha) = 2*pi*(vperp/v) since vperp/v is the radius of a

circle on the unit sphere in 3d v-space.  But the differential area

on the surface of the sphere also has a factor of differential width

dw: dA=dw*2*pi*sin(alpha).  Corresponding to delta(cos(alpha)) this

width dw of the differential band around the circle at radius

2*pi*sin(alpha) has a factor of 1/(sin(alpha)) which cancels out the

factor due to the radius of the circle.  You can convince yourself

of this with a picture of the sphere showing the relation of dw to

delta(cos(alpha))=delta(vpll/v).

You get integral[(vpll/v = -1 to 1){d(vpll/v)*2pi}] = 4pi = surface

area of unit sphere.  The contribution of each pitch zone delta(vpll/v)

is equal at delta(solid angle)/4pi = (1/nznbma) in terms of

TRANSP/NUBEAM's FBM data.

(End of discussion of delta(solid-angle)).

------------------------------------------------------------------------

Given the reconstructed TRANSP COMMON arrays (e.g. in get_fbm.for), the

number of particles of fast specie 'js' in spatial zone 'jz', is given

by:

!

! nznbma = # of vpll/v bins

! nznbmea(js) = # of energy bins

!   (in general, different fast species have different energy grids)

! bmvol(iz) = volume of iz'th spatial zone

!

znsum=0.0      ! sum to contain total number of particles this zone

!

do ia=1,nznbma

do ie=1,nznbmea(js)

dE=efbmb(ie+1,js)-efbmb(ie,js)

znsum=znsum+fbm(ie,ia,iz,js)*dE*bmvol(iz)/nznbma

enddo

enddo

Questions:  send email to dmccune (dmccune@pppl.gov).

`get_fbm' enhancements can be requested.

--------------------------------------------------------

Note by W. Heidbrink (1997)

Date: Tue, 12 Aug 1997 10:27:29 -0700 (PDT)

From: <HEIDBRINK@GAV.GAT.COM>

Subject: Beam distribution function in TRANSP

To: GREENFIEL@GAV.GAT.COM, HEIDBRINK@GAV.GAT.COM, LAZARUS@GAV.GAT.COM,

MURAKAMI@GAV.GAT.COM, TERPSTRA@GAV.GAT.COM, PETTY@GAV.GAT.COM,

FOREST@GAV.GAT.COM, STALLARD@GAV.GAT.COM, BaylorLR@ORNL.GOV

TRANSP FAST-ION DISTRIBUTION FUNCTION

W.W. Heidbrink

TRANSP calculates many quantities derived from the fast-ion distribution

function "f" but does not ordinarily save "f" itself.  Using tools

originally written by McCune and at JET, Ted Terpstra and I have created

programs to view "f" at DIII-D.

-----------------------------------------------------------------------------

1) Add lines to namelist.  You must instruct TRANSP to dump its memory

at certain selected times.  This creates a so-called ACFILE in the

appropriate result area.  Here are the lines I inserted in

71524A01TR.DAT:

!

! Dump common block:

!

!SELOUT='XXX'      !list of TRANSP common symbols to output (default=all)

SELAVG='FBM BMVOL BDENS2 EBA2PL EBA2PP' !quantities to average

AVGTIM=0.030   !averaging time (to reduce MC noise)

MTHDAVG=2      !2 for averaging after each MC timestep

OUTTIM=1.875,2.150  !up to 5 times that AC files are written

------------------------------------------------------------------------

2) Run TRANSP.  If all goes well, you'll end up with files like

$TRANSPROOT/result/D3D.91/71524A01.DATA1

$TRANSPROOT/result/D3D.91/71524A01.DATA2

with one file for each OUTTIM you selected.

-----------------------------------------------------------------------

3) Use your chosen method for accessing the fbm data; use get_fbm to

convert the data to NetCDF format if needed.

----------------------------------------------------------------------------

4) Use your chosen method for accessing neutron emission rates 2d profiles

and convert the data to NetCDF format.

----------------------------------------------------------------------------

5) Physics note:  the "fbm" in TRANSP actually has a complicated

spatial index label that moves around radially and poloidally.  There

are codes that only bother to return "f" at the outer midplane.

This nearly always is sufficient to reconstruct the full distribution

function, because almost every orbit crosses the outer midplane.

But not quite.  There are some "inner" orbits (close to the x-point

"pinch" orbit in phase space) that always have R<R_0.  These are

extremely rare for DIII-D beam ions, but could be important if we ever

pull out an enormous rf tail.

============================================================================

D. McCune:  Caution: NLFBMFLR = .TRUE. is the namelist default!!

***** See comments by MG

In NUBEAM,

NLFBMFLR = .TRUE. means: accumulate fbm(rho,theta,E,vpll/v) at particle

positions;

NLFBMFLR = .FALSE. means: accumulate fbm(rho,theta,E,vpll/v) at the orbit

guiding centers.

*****

The default is .TRUE. because of the history of use of fbm for synthetic

diagnostics code, but it is not necessarily the best choice.  In particular,

fbm has no gyro-phase dimension, and so, the accumulation of data at

particle locations that correspond to loss orbits for some gyro-phases (or

equivalently, some guiding center locations that could contribute) but

not for others, will be accumulated mostly from non-loss orbits, but, the

resultant anisotropy in the direction of contributing ions at (E,vpll/v) at

the spatial location cannot be reconstructed!

For NLFBMFLR=.FALSE., an FLR step needs to be taken to get to particle

positions e.g. for charge exchange eflux diagnostics simulation based on fbm.

Existing codes may not do this.

Of course, if the finite larmor radius is small, the switch will have little

effect.  But if it is not, e.g. in STs or other low field experiments, it is

important to consider the proper setting of NLFBMFLR, which depends on the

use to be made of the "fbm" data.

============================================================================

D. McCune:  Description of iregular 2d spatial grid over which the

"fbm" data is written...

The MCgrid, or "Monte Carlo grid", is an irregular 2d spatial grid that

was developed originally for capture of 2d binned data in a Monte Carlo

calculation (NUBEAM).  It is used for fast ion distribution functions and

for certain spatially 2d profiles output by NUBEAM-- such as, beam halo

thermal neutral sources, beam-target and beam-beam fusion rates.

The grid is aligned with flux coordinates.  It is constructed of "zone

rows" that are equally spaced in rho=sqrt(Psi_tor/Psi_tor_at_bdy); each

zone row is subdivided into a different number of zones equispaced in

the equilibrium poloidal angle coordinate theta, with fewer subdivisions

for zone rows near the axis, and more for zone rows in the edge.  The

result is a set of zones all with roughly equal volume and cross sectional

area, as is desirable for consistency of Monte Carlo summation statistics.

The number of poloidal zones per zone row is linear in the zone row index:

typically, 4 in the 1st zone row adjacent to the magnetic axis, 8 in the

next row out, 12 in the next row, etc.

There are two variants, according as the underlying MHD equilibrium is

updown symmetric, or updown asymmetric.

As internally stored, the first row layout is this for the updown SYMMETRIC

variant:

_____

/  |  \

/ 2 | 1 \

|___|___|

with theta=0 on the large major radius side; increasing to theta=pi on

the large major radius side covering the upper half of the plasma cross

section above the midplane.  The next row out would have 4 zones, the

next 6 zones, etc.

And this for the updown ASYMMETRIC variant:

_____

/  |  \

/ 4 | 3 \

|___|___|

|   |   |

\ 1 | 2 /

\__|__/

with theta=-pi on the lower branch on the small major radius side, theta=0

on the large major radius side, and theta=pi on the upper branch on the small

major radius side.  The next zone row contains 8 zones, the next 12 zones,

and so on.

Poloidal zones are stored contiguously, with the poloidal zone index

increasing with increasing theta coordinate, theta being oriented

counter-clockwise in the plasma cross section drawn to the right of

the machine axis of symmetry.

--------------------------------------------------------

Spatial resolution of fbm output from TRANSP

There are two relevant TRANSP namelist controls:

NZONE_NB  (default =20) -- base radial resolution for NUBEAM

rule: divisible by 10

NZONE_FB  (default =10) -- radial resolution for fbm

rule: NZONE_FB an even divisor of NZONE_NB

Thus for NZONE_NB=20, legal values of NZONE_FB are 5, 10, and 20.

For NZONE_NB=40, legal values of NZONE_FB are 5, 10, and 20 and 40.

The number of poloidal zones per radial row depends on the radial row index:

NZONE_FB = 10

row index           updown symmetric        updown asymmetric

1                      2                       4

2                      4                       8

3                      6                      12

4                      8                      16

5                     10                      20

6                     12                      24

7                     14                      28

8                     16                      32

9                     18                      36

10                     20                      40

-----                   -----

total                  110                     220

NZONE_FB = 20

arithmetic progression extended through 20 zone row indices.  The total

number of zones is NZONE_FB*(NZONE_FB+1) for the updown symmetric case, and

twice that for the updown asymmetric case: 420 and 840, respectively, for

NZONE_FB = 20.

Some variants of the MC grid include extra zone rows that extend beyond

the plasma boundary.